UCSC Extension – Digital Design with FPGA

Jae-Yang Park (jaeyangp@gmail.com)

Problem 2.6.1

- Show that lutmask 8000 implements 4-inputs AND gate

| a | b | c | d | F = a b c d LUTMASK = 8000 F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

- Implement a 4 inputs XOR gate using a LUT4

| a | b | c | d | F = a’b’c’d +a’b’cd’ +a’bc’d’ +a’bc’d’ +ab’c’d’ +ab’cd +abc’d+abcd’ LUTMASK = 6996 F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |

- Implement the function below using a LUT4

| a | b | c | d | F = abcd + a’b’c’d’ + ad’ + a’bc’ + b’d + a’bc’ + abc’d + bcd + a’cd’ = TAUTOLOGY LUTMASK = FFFF F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Problem 2.6.2

- Implement all the 16 possible two-input functions using the above block

Complete the table as follows:

|  |  | a0 | a1 | sa | b0 | b1 | sb | s0 | s1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | XOR (a, b) | 0 | 1 | b | 1 | 0 | b | a | 0 |
| 2 | AND (a, b) | 0 | 0 | b | 0 | 1 | b | a | 0 |
| 3 | OR (a, b) | 0 | 1 | b | 1 | 1 | b | a | 0 |
| 4 | NAND (a, b) | 1 | 1 | b | 1 | 0 | b | a | 0 |
| 5 | NOR (a, b) | 1 | 0 | b | 0 | 0 | b | a | 0 |
| 6 | XNOR (a, b) | 1 | 0 | b | 0 | 1 | b | a | 0 |
| 7 | A < B | 0 | 1 | b | 0 | 0 | b | a | 0 |
| 8 | A > B | 0 | 0 | b | 1 | 0 | b | a | 0 |
| 9 | A LE B | 1 | 1 | b | 0 | 1 | b | a | 0 |
| 10 | A GE B | 1 | 0 | b | 1 | 1 | b | a | 0 |
| 11 | BUFFER (A) | 0 | 0 | b | 1 | 1 | b | a | 0 |
| 12 | BUFFER (B) | 0 | 1 | b | 1 | 1 | b | a | 0 |
| 13 | NOT (A) | 1 | 1 | b | 0 | 0 | b | a | 0 |
| 14 | NOT (B) | 1 | 0 | b | 1 | 0 | b | a | 0 |
| 15 | CONTRIDICTION | 0 | 0 | b | 0 | 0 | b | a | 0 |
| 16 | TAUTOLOGY | 1 | 1 | b | 1 | 1 | b | a | 0 |

2. Implement f = ab + b’c + d using the above block

| a | b | c | d | ab | b’c | ab+b’c+d |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 0 | 1 |

|  |  | a0 | a1 | sa | b0 | b1 | sb | s0 | s1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | AND (a, b) | 0 | 0 | b | 0 | 1 | b | a | 0 |
| 3 | OR (a, b) | 0 | 1 | b | 1 | 1 | b | a | 0 |

OR

ab+b’c

d

ab+b’c+d

OR

ab

b’c

ab+b’c

AND*

c

b

b’c

AND

a

b

ab

ab+b’c+d

0

ab+b’c

d

d

0

1

1

1

0

0

ab+b’c

0

ab

b’c

b’c

1

1

1

b’c

0

0

0

b

c

1

c

ab

b

0

a

b

0

1

0

0

0

1

0

1

0

1

OR

0

1

0

1

0

1

OR

0

1

0

1

0

1

OR

0

1

0

1

0

1

OR

AND

a

b

ab

AND*

c

b’c

OR

ab+b’c

OR

d

ab+b’c+d

d

Problem 3.7.1

| SW5 | SW4 | SW3 |  |
| --- | --- | --- | --- |
| OFF | OFF | OFF |  |
| OFF | OFF | ON |  |
| OFF | ON | OFF |  |
| OFF | ON | ON |  |
| ON | OFF | OFF |  |
| ON | OFF | ON |  |
| ON | ON | OFF |  |
| ON | ON | ON |  |

Problem 3.7.1

| SW6 | SW5 | SW4 | SW3 |  |
| --- | --- | --- | --- | --- |
| OFF | OFF | OFF | OFF |  |
| OFF | OFF | OFF | ON |  |
| OFF | OFF | ON | OFF |  |
| OFF | OFF | ON | ON |  |
| OFF | ON | OFF | OFF |  |
| OFF | ON | OFF | ON |  |
| OFF | ON | ON | OFF |  |
| OFF | ON | ON | ON |  |
| ON | OFF | OFF | OFF |  |
| ON | OFF | OFF | ON |  |
| ON | OFF | ON | OFF |  |
| ON | OFF | ON | ON |  |
| ON | ON | OFF | OFF |  |
| ON | ON | OFF | ON |  |
| ON | ON | ON | OFF |  |
| ON | ON | ON | ON |  |